Happy Birthday Greeting Device

Project Documentation

Platform: ESP32  |  Display: SH1106 128×64 OLED (I2C)

Input: Capacitive Touch Sensor  |  Output: Passive Buzzer + Animation

Framework: Arduino (PlatformIO / Arduino IDE)

| Component | Detail |
| --- | --- |
| Microcontroller | ESP32 Dev Board |
| Display | SH1106 128×64 I2C OLED |
| Input | Capacitive Touch Sensor (GPIO 4) |
| Output | Passive Buzzer (GPIO 5) + OLED |
| Library | U8g2, Wire, Arduino Core |
| Melody | Happy Birthday to You — 140 BPM |

# Table of Contents

1.   Project Overview

2.   Hardware Requirements

3.   Pin Configuration

4.   Library Dependencies

5.   System Architecture

6.   Module Breakdown

6.1   Music Engine

6.2   Animation Engine

6.3   Display Rendering

6.4   State Control

7.   Data Structures

8.   Function Reference

9.   Configuration & Customization

10.   Timing & Performance

11.   Known Limitations & Improvements

# 1. Project Overview

This project implements an interactive Happy Birthday greeting device using an ESP32 microcontroller. When a user touches the capacitive touch sensor, the device:

- Plays the "Happy Birthday to You" melody on a passive buzzer.

- Displays a real-time OLED animation featuring scrolling text, floating balloons, and twinkling sparkles.

- Reverts to an idle screen when the touch is released, stopping all sounds immediately.

The entire animation and music playback system is non-blocking, using millis()-based timing to allow simultaneous music and animation updates without delay() stalls.

# 2. Hardware Requirements

| Component | Specification |
| --- | --- |
| Microcontroller | ESP32 (any 38-pin dev board) |
| OLED Display | SH1106 128×64, I2C interface |
| Touch Sensor | Capacitive touch module (HIGH when touched) |
| Buzzer | Passive buzzer (supports tone()) |
| Wiring | Jumper wires, breadboard |
| Power | 3.3V / 5V via USB |

Note: The SH1106 is similar to the common SSD1306 but uses a slightly different driver. Ensure you use the U8G2_SH1106 constructor, not U8G2_SSD1306.

# 3. Pin Configuration

| GPIO Pin | Connected To | Direction |
| --- | --- | --- |
| GPIO 4 | Touch Sensor OUT | INPUT |
| GPIO 5 | Buzzer (+) terminal | OUTPUT |
| GPIO 21 | OLED SDA | I2C (bidirectional) |
| GPIO 22 | OLED SCL | I2C (output) |

Note: GPIO 21 (SDA) and GPIO 22 (SCL) are the default hardware I2C pins on the ESP32. The Wire.begin(21, 22) call in setup() explicitly sets these.

# 4. Library Dependencies

| Library | Purpose | Install Via |
| --- | --- | --- |
| U8g2lib | OLED display driver (SH1106) | Arduino Library Manager / PlatformIO |
| Wire | I2C communication | Built-in (Arduino core) |
| Arduino.h | Core functions (tone, millis, etc.) | Built-in |

### U8G2 Constructor Used

U8G2_SH1106_128X64_NONAME_F_HW_I2C u8g2(U8G2_R0, U8X8_PIN_NONE);

F = Full buffer mode (1024 bytes RAM).  HW_I2C = Hardware I2C.  U8G2_R0 = No display rotation.

# 5. System Architecture

The firmware follows a two-state event-driven loop. The wasHeld boolean tracks the previous frame's touch state to detect rising and falling edges cleanly.

loop()

|

+-- Read TOUCH_PIN

|

+-- held == HIGH --> CELEBRATION STATE

|                     +-- startMusic()      (on rising edge)

|                     +-- resetAnimation()  (on rising edge)

|                     +-- updateMusic()     (every iteration)

|                     +-- drawCelebration() (every 40 ms)

|

+-- held == LOW  --> IDLE STATE

+-- noTone()          (on falling edge)

+-- drawIdle()        (on falling edge)

### State Transition Table

| Current State | Condition | Next State | Action |
| --- | --- | --- | --- |
| IDLE | Touch detected (rising edge) | CELEBRATION | resetAnimation(), startMusic() |
| CELEBRATION | Touch held | CELEBRATION | updateMusic(), drawCelebration() |
| CELEBRATION | Touch released (falling edge) | IDLE | noTone(), drawIdle() |

# 6. Module Breakdown

## 6.1  Music Engine

The music engine plays Happy Birthday to You as a non-blocking looping melody. Notes are defined as frequency constants in Hz. Durations are in beats relative to BPM tempo. REST = 0 represents a silent pause. Tempo is fixed at 140 BPM.

### Beat-to-Millisecond Conversion

float bpmToMs(float beats) {

return beats * (60000.0 / TEMPO);

}

// At 140 BPM: one beat = 428.57 ms

### Note Articulation (15% Gap)

Each note is silenced at 85% of its duration before the next note starts, creating a natural articulation gap that prevents notes from blending together.

if (noteOn && elapsed >= (unsigned long)(noteDurMs * 0.85)) {

noTone(BUZZER_PIN);

noteOn = false;

}

### Music State Variables

| Variable | Type | Description |
| --- | --- | --- |
| noteIndex | int | Current position in the melody array |
| noteStart | unsigned long | millis() timestamp when current note began |
| noteDurMs | unsigned long | Duration of current note in milliseconds |
| noteOn | bool | Whether the buzzer is actively sounding |

## 6.2  Animation Engine

The animation runs at ~25 FPS (one frame every 40 ms), controlled by FRAME_MS = 40. Three sub-systems run in parallel each frame: Balloons, Sparkles, and Scrolling Text.

### Balloons

Three balloons float upward independently. Each frame, y decreases by speed. When y < -20 (off the top of screen), it wraps back to y = 72 (below the screen bottom). Each balloon is drawn as an ellipse (7x9 px) with a short wavy string below it.

Balloon balloons[3] = {

{20,  70, 1},   // slow

{64,  90, 2},   // fast

{105, 50, 1},   // slow, starts higher

};

### Sparkles

Eight sparkles are distributed across the screen, each cycling through a 6-state blink pattern. The phase offset per sparkle ensures they don't all blink in sync, creating a natural twinkling effect.

| State | Pattern |
| --- | --- |
| 0 | Single pixel |
| 1 - 2 | Small plus (+) |
| 3 | Large star (3x3 diamond) |
| 4 - 5 | Off (invisible) |

### Scrolling Text

The "Happy Birthday!" text scrolls from right to left at 2 pixels per frame (~50 px/sec). Two copies are drawn 140 px apart for a seamless infinite scroll — when Copy 1 exits the left edge, Copy 2 takes its place and the offset resets.

textX -= 2;

if (textX <= -140) textX += 140;  // seamless wrap

## 6.3  Display Rendering

### Idle Screen — drawIdle()

Displays a static prompt with "Touch to wish" (helvR10 font) and "Happy B-Day!" (helvB12 bold). A small circle with a line below acts as a visual touch-here indicator.

### Celebration Screen — drawCelebration()

| Step | Action |
| --- | --- |
| 1 | Clear buffer |
| 2 | Draw 8 sparkles (position + phase-based blink state) |
| 3 | Draw 3 balloons at current Y positions |
| 4 | Draw scrolling 'Happy Birthday!' bold text at y = 38 |
| 5 | Draw scrolling star decoration line at y = 50 |
| 6 | Send buffer to OLED display |
| 7 | Advance all animation state variables |

## 6.4  State Control

### setup() sequence

1. pinMode(TOUCH_PIN,  INPUT)

2. pinMode(BUZZER_PIN, OUTPUT)

3. Wire.begin(21, 22)   // I2C on GPIO 21/22

4. u8g2.begin()

5. u8g2.setContrast(200)

6. drawIdle()

### loop() sequence

1. Read touch state  (held = HIGH / LOW)

2. If held:

a. If rising edge  --> resetAnimation() + startMusic()

b. updateMusic()       (advance melody, non-blocking)

c. If 40ms elapsed --> drawCelebration()

3. If not held:

a. If falling edge --> noTone() + drawIdle()

4. wasHeld = held

5. delay(5)               // debounce / yield

# 7. Data Structures

## Balloon

struct Balloon {

int x;      // horizontal position (px)

int y;      // vertical position (px), decreases each frame

int speed;  // pixels per frame (upward velocity)

};

## Sparkle

struct Sparkle {

int x;      // fixed horizontal position (px)

int y;      // fixed vertical position (px)

int phase;  // blink cycle offset (0-3), prevents sync

};

# 8. Function Reference

| Function | Parameters | Returns | Description |
| --- | --- | --- | --- |
| bpmToMs() | float beats | float | Converts beat count to ms at current tempo |
| startMusic() | — | void | Resets melody to note 0 and begins playback |
| updateMusic() | — | void | Advances melody state machine (non-blocking) |
| drawBalloon() | int x, y | void | Draws one balloon at given coordinates |
| drawSparkle() | int x, y, phase | void | Draws sparkle based on global tick + phase |
| drawCelebration() | — | void | Renders one animation frame; advances state |
| drawIdle() | — | void | Renders the static idle screen |
| resetAnimation() | — | void | Resets all animation variables to initial values |
| setup() | — | void | Pins, I2C, display initialization |
| loop() | — | void | Main event loop: touch input drives music and display |

# 9. Configuration & Customization

All key parameters are defined as constants at the top of the file for easy tuning.

| Parameter | Default | Effect |
| --- | --- | --- |
| TOUCH_PIN | 4 | GPIO for touch sensor input |
| BUZZER_PIN | 5 | GPIO for passive buzzer output |
| FRAME_MS | 40 | Frame interval in ms (~25 FPS). Lower = faster animation |
| TEMPO | 140 | Melody speed in BPM |
| textX scroll step | 2 px/frame | Scroll speed. Increase for faster scrolling text |
| Balloon speed | 1 or 2 px/frame | Per-balloon upward velocity |
| setContrast() | 200 (0-255) | Display brightness. Higher = brighter OLED |

### Changing the Melody

Replace the melody[] and noteDurations[] arrays. Both arrays must have the same number of elements. The noteCount is calculated automatically:

const int noteCount = sizeof(melody) / sizeof(melody[0]);

# 10. Timing & Performance

| Parameter | Value |
| --- | --- |
| Loop cycle (minimum) | ~5 ms (due to delay(5)) |
| Animation frame rate | ~25 FPS (40 ms / frame) |
| Music update granularity | Every loop cycle (~5 ms) |
| One beat at 140 BPM | ~428.57 ms |
| Note articulation gap | 15% of note duration |
| Display buffer mode | Full frame buffer (128x64 / 8 = 1024 bytes RAM) |

Note: The delay(5) in loop() acts as a soft debounce for the touch sensor. It is safe because music and animation timing is handled independently via millis().

# 11. Known Limitations & Improvements

| # | Issue | Suggested Fix |
| --- | --- | --- |
| 1 | Touch debounce is minimal — rapid noise can trigger false starts. | Require N consecutive HIGH reads before treating as pressed. |
| 2 | Melody loops indefinitely while touch is held — no end-of-song event. | Add a one-shot mode flag that stops music after one full play. |
| 3 | Text wrap offset (140px) is hardcoded — may not match actual font width. | Measure text width with u8g2.getStrWidth() and use it dynamically. |
| 4 | No I2C error handling — missing OLED may cause u8g2.begin() to hang. | Check return value of begin() and blink the built-in LED on error. |
| 5 | Balloon positions reset identically on every touch — no randomness. | Seed balloon Y values with esp_random() on each new touch. |
| 6 | Display contrast is fixed — no adaptation to ambient light. | Add an LDR and map its reading to setContrast() dynamically. |